America’s No. 1 Mast 

Manufacturer
U.S. SPARS, INC.

Gainesville, Florida


03084105 Manual furling mast

	US SPARS, INC

	6320 NW 123RD PLACE 

GAINESVILLE, FL

32653

	TEL: 386-462-3760  or 800-928-0786
F AX: 386-462-3448 

WEB: WWW.USSPARS.COM


CONGRATULATIONS!

YOU NOW OWN THE MOST WIDELY USED MAINSAIL FURLING SYSTEM IN

THE WORLD.

THE Z SPAR MAINSAIL FURLER IS USED ON NEARLY ALL THE WORDS TOP

PRODUCTION BOATS; IT HAS PROVEN ITS SELF TO BE THE SYSTEM YOU NEED

WHEN QUALITY AND SERVICES ARE AN ISSUE.

ZSPAR HAVE BEEN MAKING SPARS FOR 30 YEARS, OUR PRODUCTION FACILITY

AROUND THE WORLD USE THE VERY LATEST TECHNIQUES IN SPAR

PRODUCTION. WHICH ALLOW US TO PRODUCE OVER 6000 MASTS A YEAR.

IF YOU HAVE ANY SET UP OR OPERATIONAL QUESTION ABOUT ANY PART OF

YOUR Z SPAR RIG YOU CAN CONTACT OUR TECHNICAL DEPARTMENT WHICH

WILL BE ABLE TO GUIDE YOU THROUGH ANY ASPECT OF OUR RIGS.

WE CARRY A VAST ARRAY OF SPARE PARTS FOR ALL Z SPAR PRODUCTS, SO YOU

CAN COME DIRECTLY TO THE MANUFACTURER IF YOU REQUIRE ANY P ARTS.

NO MIDDLE MAN TO PAY, JUST A VERY GOOD PRICE AND SERVICE. THAT'S THE

US SPARS WAY.

ALL THAT’S LEFT NOW IS TO SET SAIL AND ENJOY PEACE OF MIND SAILING

WITH Z SPAR ON BOARD!

Page 1

Z Spar Furling Mast

Installing The Outhaul Rope:

This rope attaches to the front of the boom traveler, it leads around the clew block in the mainsail, underneath the pulley on top of the boom traveler, around the sheave in the boom outhaul end casting, through the halyard exit under the boom (or over the sheave in the front end of the boom) through a pulley behind the kicker and forward to a swivel pulley at the base of the mast. This leads aft in the same way as the furling rope.

Installing The Furling Rope:

Inside the drum underneath the gooseneck it is possible to see the coils of rope wound on the furling drum. Make sure the rope is fully wound on the drum then unwind three turns. Lead the rope from this slot around the pulley situated under the boom just in front of the kicker/boom vang fitting, then through the swivel pulley attached to the mast base and back to a self-tailing winch on the coach roof via the halyard organizer.

Installing The Mainsail:

Open and remove all four inspection covers. At the top set of openings you will see the sail entry slot cut in the furling internal foil extrusion. Feed the head of the sail through the mast slot and into the sail entry slot in the foil. Lower the mail halyard down to allow the shackle on the halyard swivel to be attached to the strap sewn into the head of the sail using an allen key.

Continue to feed the sail into the slot whilst the main halyard is being hoisted. Do not attempt this with the wind astern. (This task is best attempted in little or no wind). Once the sail is hoisted to a point where the tack web loop is almost level with the large tack shackle stop. You now have to back the remaining luff down the foil form the sail entry until you can connect it to the tack shackle. It is important to have the full length of the luff in the foil form shackle to shackle with no cut away below the sail entry. Connect the bottom web of the sail to the base of the furling extrusion with the shackle provided, by gaining access through the lower inspection holes. Please note that this tack shackle also connects the furling extrusion to the furling drum. Refit inspection covers. Complete the installation by applying moderate tension to the main halyard.

Furling the Sail:

Hoist the boom topping lift or ease the kicker so that the leach has little or no tension.

Maintaining a slight tension on the outhaul, furl the sail with the wind ahead. (A slight pressure from the wind will prevent creases in the sail). Insure that the furling drum has two or three turns of rope left on it when the sail is fully furled. The sail will only furl as far as the reinforcement patch.

Changing the Furling Rope:

Open the lower inspection covers and remove the tack shackle. This will disconnect the furling drum from the furling foil. Remove the machine screws holding the furling mechanism to the mast. These are located two at the top of the drum and two at the bottom of the drum. On some models there will be two additional screws or rivets on the side of the mast to be removed. Using a large screwdriver, leaver the furling drum away from the mast at the bottom end. The furling rope is retained by a simple knot inside the furling screw push the rope towards the drum and the knot will appear at the bottom of the furling screw. Undo the knot remove rope and replace. We recommend a 10mm braid on our larger gears (Beneteau 381 to 461) and an 8mm on our smaller gears (Beneteau 331) this braid must be of a good quality that will not flatten. This operation is made easier with the sail removed.

Maintenance: 

Z spar furling masts require minimum maintenance.

The boom traveler should be flushed with detergent and fresh water regularly.

The furling rope should be replaced every four years or as required. Clean and flush top and bottom of furling mechanism regularly and spry with WD 40 or Harken lube (the ball bearings in the furling mechanism and halyard swivel are all stainless steel so need minimum maintenance).
Remove mainsail every year for inspection and every 3-4 years it is recommended that you let your local sailmaker inspect and service the sail.
Trouble Shooting:

When unfurling the sail if there is resistance for the boom traveler to move, the most likely course will be excessive mainsheet or vang/kicker tension. Check also for friction with in the halyard organizers or at the mast base blocks, if these do not move freely it will cause slowing of the gear.

If the sail is not new there may be localized hardening of the head reinforcement, or fraying at the leach, which can cause friction. It is worth having the sail checked every year to avoid such problems.

When unfurling the sail if there are creases originating at the luff, and if these cannot be removed by increasing the main halyard tension, the most likely cause is that the sail maker has made the luff too long, (too much halyard tension will also cause friction at the halyard swivel).

Alternatively, the sail may have been furled with too much kicker tension.
With any furling mast it is important that pre bend be kept to a minimum, although a small amount of bend can help stabilize the middle of the mast the straighter a furling mast is the better it will work, please refer to our tuning guide for  help with mast set up.
If you are having ongoing problems with furling it is a good idea to remove the sail and then try turning the furling system to establish if your problem stems from the furling system or the sail. If you find the system works well without the sail it is advisable to have a sailmaker check the sail. We recommend Neil Pryde Sails Int, 203-874-6984 as a very reliable sailmaker who has a good knowledge and experience with our system.

If you find the system is not turning as free as it should then removing the drum is advisable, with the drum removed you can clean and inspect it for damage. If you have time you can send the drum to our facility in Florida where we can service the system and replace any necessary parts. The base cost for this service is approximately $130.00 plus return shipping.

It is necessary to change to furling inhaul line regularly; this line can harden and flatten after just a season or two. If this line is not inspected and replace when required it can jump the grooves in the drum and damage the back plate. We recommend the use of a good quality line with high abrasion resistance, your system will come fitted with metric line but if you have to use imperil line then replace 8mm with 5/16” and 10mm with 3/8”.

[image: image1.png]



[image: image5.png]SAILMAKERS INFORMATIONS

P 3 2 I L A A A A A A A R A A
sosn 106 12545 170102 2690 1|61 01 s |51 51 |0 80 001 | o)
B NARARARZEARR NI
ol [ fie|1e| [l1e] |® oo | o
s |85 s fala|sls|sllaloldfoolo
e e e el S AN
A
om | A0 OO E]F ] o | sw
T f
s | 5 fs | 8|00 som
anary
T
w40 || w0z snn

FATEON R R Y Y 0 N 7S
7201 [ 50 [ w0 5 [ 70|10 [0 som [ sz
2380 [ 60 5 [ 7 [100[200 [0 som [ 10z
a0 |60 | 3 [ 25 [ 100] 200] 260 | 1z | 5015
Too07 | 5 | 40 | 15 [ 110] 260 | 280 | o | w012
Teo0s | 7% |~ aw] w0 - B
o0 | %[~ (2015 -

Farig sor | swrsion
ot rock_| iyt
7250 [ 60 10| 1o0[8o| @l 20 (46| e |

7300¢ |60 [s00] o]0 eoo] 20 [ a0 [r0a| o | 7ase
7 a00¢ [ 60 [san[vao[aco[ eou] 5o [0 [1o0| oo | v
T500¢ [0 [600[100]400] 600[ 3 [0 [100] 7rs | >
Z600¢ [0 5001004001 000[ 35 [0 [100] 7rs | ot
Z700¢ [0 500100 50]1000[ 3 [0 [100] s1s | kot
a0t 60 [500]100[50] 1000 35 [0 [100] s | ko
9007 00 [500[100]50]1 200] 3 [ 0 [100] s | o1
Z1100¢] 60 [500[100[d50 1 600[ 35 [0 [100] s | o1
77200 60 500100501 500[ 35 [ 80 [100] e | wer

RECOUNANDATONS FOR FRLNG WIST

ot use atens on main sai. ¥ 3 .I
i 1
i —— |} CER





[image: image2.png]MAST FURLING MECHANISM

etin 230 | a00 | 400 | 500 | 600 | 700 | soo | a00 | vico| raco) 77 | 1i0

266|256 | 255 | 258 | 255 | 288 | 259 | 259 | 258 | 258

3164 a1es | aies | es| 165 | 168 | aree| 3605 | 3167 3168

] o sezs| seos s217| 2214
o |
ezr
pagmeesn| | 326 o626 | o626 aezs| aeas| aezs| | ae2r| aezr| seza) 5139
sz o oz

s | 3247 3047 247 327 | 3247 | aaan| 27| azar| sass| azee

3208|3208 3208 320 3208 3208 3208 3208 3208 3208 3208 3208
v st

16| 3108] 3168| a168| 3108| aea] tes| a1es| 1502|1662 3tes 3168

Hoyadswie!
sickls






[image: image3.jpg]







[image: image6.png]Boom car loading system

Congratulations on gece ving your new model 1 § Spazs TORLON ball bearing boom car. The car hax
£ome o it orn Ineding track, s Loading track i similar o the boom rack o you baom, but urlike
your boom frack the oading frack has ro T piece for fxing o the boom section, You will wofice a pisce
aF fape at each erc ofthe Iuding trc , s i t revent the car from slding offthe frack ard you
\oolking every where for te bearings, s fhere are rof capive bearings. When you e ready o el the
cur st remave the exd cap at the FORWARD erd of your boom track, then r2mave the old boom car.
Vou can then carefully remave the fape from the exd of e track clotest o the sheave on the boom car.
Place the track on top of your boorm with the loading track in confact wih the forward exd of the boom
ruck, yon cun then simpy and very care fully slic the car from the loading track fo your boom k.

F you have any diffculty with the instalstion of your newrboom car please do not hesiate o sall our
technical depariment who will be moe than happy o alk you Buogh eny pertof the process





[image: image4.png]



U.S. SPARS, Inc.

6320 N.W. 123rd Place

Gainesville

Florida

32653
Phone (386) 462-3760 Fax (386) 462-3448

Web: www.usspars.com
Z.SPARS,UK,LTD

Unit 2 Pond Hall Rd.

Hadleigh,

Suffolk, IP7 5PH

England

Tel:(44)1473822130 Fax(44)1473827354

www.zsparsuk.com 

PERFORMANCE SPARS

10 Bis, Avenue, Louis-Lumiere

17184 Perigny Cedex

France

Phone (33) 0546454202  Fax (33) 0546447631

Our People Make The Difference

� INCLUDEPICTURE "http://www.usspars.com/USSparManual_files/sailmakersinformations.gif" \* MERGEFORMATINET ���





� INCLUDEPICTURE "http://www.usspars.com/USSparManual_files/boomcar.gif" \* MERGEFORMATINET ���








